Multi-dimensional Modeling of Ore Bodies

Making Sense of Empirical Data

By: Laurie Tomczyk

Due: July 30, 2004

Part One: Michigan Standards Addressed

Mathematics/Strand V/Content Standard 2/High School
Benchmark 4
Analyze problems that can be modeled by functions, determine strategies for solving the problems and evaluate the adequacy of the solutions in the context of the problems.

Overview:
Students learn analytic thinking most effectively when it is studied in the context of problems and applications.

Lesson Goals: 
Modeling involves identifying and selecting relevant features of a real-world situation, representing those features symbolically, analyzing and reasoning about the model and the characteristics of the situation, posing a model, and considering the accuracy and limitations of the model. In this activity students will study modeling in greater depth, generating and using data, to model ore body locations and generate a three-dimensional model of the area.

Mathematics/Strand III/Content Standard 3/High School
Benchmark 5
Employ investigations, mathematical models and simulations to make inferences and predictions to answer questions and solve problems.

Overview: We live in a sea of information. In order not to drown in the data that inundate our lives every day we must be able to process and transform data into useful knowledge. The ability to interpret data and to make predictions and decisions based on data is an essential basic skill for every individual.

Lesson Goals:

The students will analyze their model and make proposals as to where the mining company should sink a well for production.

Science/Strand V/Content Standard 4/High School
Benchmark 4
Explain how technology and scientific inquiry have helped us learn about the earth and universe.

Overview: The contemporary applications of mathematics in virtually every field of work and study rely on algebraic and analytic thinking and communication as fundamental tools.

Lesson Goals: The remoteness of objects in the earth and universe necessitates the use of sophisticated technologies to make even basic observations. Advancements in technology in mining include computer imaging/modeling, spectroscopes, and charged-coupled devices. The three dimensional computer simulation of the mine received in the Teachers’ Earth Science Institute 2004 will be used to demonstrate to the students how a 3-D model was actually used in a mining operation.

Part Two: The Lesson Plan

Using Drill Core Sample Data

To Make Decisions about Mine Management

Day One: Generate Two-Dimensional Models of a Geologic Cross Section (One complete 55 minute class session.)

Objectives: The students will identify four different rock types in the strata and use this identification and data to construct a two dimensional geologic cross-section.

Materials:

Drill core samples	Data tables indicating depth and angles of the core samples

Large graph paper	Colored pencils		Straight edge

Protractor		Twelve transparencies		Calculator

Procedure:

- First determine the type of rock in each core sample. (2 points each)

  - Unit A:__________________

  - Unit B:__________________

  - Unit C:__________________

  - Unit D:__________________

  - Unit E:__________________

- Determine a scale for the depth of the drill data. Make a y-axis. (5 points)

- Assume the scale is completely horizontal. Determine a scale for the distance between drill holes. (5 points)

- Mark the location of the drill holes on the graph paper. (10 points)

- Mark the depths where the rock units meet under their respective drill hole for all five rock units. See example below. (40 points)

For Example: (*Your answer goes here)

Example: Core 80E (Drill hole is 80 ft east from baseline.)

| Unit | Rock Type | Depth |
| --- | --- | --- |
| A | *Massive Basalt | 0-10 |
| B | Brachiated Basalt | 10-40 |
| C | Conglomerate | 40-50 |
| D | Brachiated Basalt | 50-80 |
|  |  | Bottom of hole |

Drill Hole

Massive Basalt

Brachiated Basalt

Conglomerate

5

10

15

20

25

30

35

40

45

50

55

60

65

70

75

80

- Connect the dots between similar contacts in all the drill hoes. Determine a    legend and color in the regions suggested by the drill core data. (10 points)

- Conclusion: Based on your cross-section, describe a possible geological progression of the deposition of rock layers. (20 points)

Drill Core Data

Core 0E

| Unit | Rock Type | Depth |
| --- | --- | --- |
| A |  | 0-10 |
| B |  | 10-50 |
| A |  | 50-180 |
| D |  | 180-200 |
| E |  | 200-270 |
|  |  | Bottom of hole |

Core 200E

| Unit | Rock Type | Depth |
| --- | --- | --- |
| A |  | 0-45 |
| B |  | 45-85 |
| A |  | 85-215 |
| D |  | 215-235 |
| E |  | 235-300 |
|  |  | Bottom of hole |

Core 600E

| Unit | Rock Type | Depth |
| --- | --- | --- |
| B |  | 0-5 |
| A |  | 5-120 |
| B |  | 120-160 |
| A |  | 160-290 |
| D |  | 290-310 |
| E |  | 310-400 |
|  |  | Bottom of hole |

Core 900E

| Unit | Rock Type | Depth |
| --- | --- | --- |
| B |  | 0-60 |
| A |  | 60-175 |
| B |  | 175-215 |
| A |  | 215-345 |
| D |  | 345-365 |
| E |  | 365-400 |
|  |  | Bottom of hole |

Day Two: Generate a Three-Dimensional Model of a Geological Cross Section (extended lab day: 130 minute session)

Objectives: The students will use data tables to construct a three dimensional geologic cross-section.

Pre-lab Discussion:

- Moving the drilling rig is very expensive. How can mining companies minimize costs of drilling?

  - Diagonal drilling, both at surface and below ground.

  - How could the model be modified so that the angles could be taken into account?

- What do mining companies do when they find a possible ore body?

  - Considerations for an Underground Copper Mine

    - Lowest mineable grade = 1% copper

    - Ore body must be greater than 15 feet thick.

    - Actual minimum cutoffs of ore grade depend on tonnage and economics. Open mines can have much lower ore grade cutoffs than underground mines.

Procedure:

- Match baseline for data to the Plexiglas baseline.

- Draw cross sections on large graph paper

- Transfer sections from paper to Plexiglas sheets. Use the following code for rock types.

  - Green ///// - Amygdaloidal Basalt

  - Yellow /////- Conglomerate

  - Blue ///// - Massive Basalt

  - Red- Conglomerate or amygdaloidal basalt with possible ore

    - Use + + + if grade of ore is more than 1%.

    - Use o o o  if grade of ore is less than 1%.

From the analysis of these cross sections they will make a proposal to the mining company as to where to begin extraction for ore in the ore body.

PLATE #1 Drill Core Data

DRILLHOLE #1, Vertical, 100W

| Depth | Rock Type | Assay |
| --- | --- | --- |
| 0-15 | Conglomerate | 0 |
| 15-35 | Amygdaloidal Basalt | 0 |
| 35-140 | Massive Basalt | 0 |
| 140-175 | Conglomerate | 0 |
| 175-180 | Amygdaloidal Basalt | 0 |
| 180-215 | Massive Basalt | 0 |
| 215-235 | Amygdaloidal Basalt | 0 |
| 235-315 | Massive Basalt | 0 |
| 315-325 | Amygdaloidal Basalt | 0 |
| 325-345 | Massive Basalt | 0 |
| Bottom of hole |  |  |

DRILLHOLE #3,  77◦ W, 150E

| Depth | Rock Type | Assay |
| --- | --- | --- |
| 0-10 | Amygdaloidal Basalt | 0 |
| 10-70 | Massive Basalt | 0 |
| 70-100 | Amygdaloidal Basalt | 0 |
| 100-195 | Massive Basalt | 0 |
| 195-260 | Conglomerate | 0 |
| 260-280 | Amygdaloidal Basalt | 0 |
| 280-390 | Massive Basalt | 0 |
| 390-395 | Amygdaloidal Basalt | 0 |
| 395-435 | Massive Basalt | 0 |
| 435-450 | Amygdaloidal Basalt | 0 |
| Bottom of hole |  |  |

DRILLHOLE #2,  60◦ W, 150E

| Depth | Rock Type | Assay |
| --- | --- | --- |
| 0-5 | Amygdaloidal Basalt | 0 |
| 5-55 | Massive Basalt | 0 |
| 55-80 | Amygdaloidal Basalt | 0 |
| 80-165 | Massive Basalt | 0 |
| 165-190 | Conglomerate | 0 |
| 190-210 | Amygdaloidal Basalt | 0 |
| 210-290 | Massive Basalt | 0 |
| 290-315 | Conglomerate | 0 |
| 315-320 | Amygdaloidal Basalt | 0 |
| 320-345 | Massive Basalt | 0 |
| 345-365 | Amygdaloidal Basalt | 0 |
| 365-420 | Massive Basalt | 0 |
| 420-430 | Amygdaloidal Basalt | 0 |
| 430-450 | Massive Basalt | 0 |
| Bottom of hole |  |  |

DRILLHOLE #4 Vertical, 150E

| Depth | Rock Type | Assay |
| --- | --- | --- |
| 0-10 | Amygdaloidal Basalt | 0 |
| 10-80 | Massive Basalt | 0 |
| 80-110 | Amygdaloidal Basalt | 0 |
| 110-225 | Massive Basalt | 0 |
| 225-265 | Conglomerate | 0 |
| 265-285 | Amygdaloidal Basalt | 0 |
| 285-415 | Massive Basalt | 0 |
| 415-420 | Amygdaloidal Basalt | 1.8% Cu |
| 420-480 | Massive Basalt | 0 |
| 480-500 | Amygdaloidal Basalt | 0 |
| 500-595 | Massive Basalt | 0 |
| 595-605 | Amygdaloidal Basalt | 0 |
| 605-615 | Massive Basalt | 0 |
| Bottom of hole |  |  |

DRILLHOLE #5, Vertical, 280E

| Depth | Rock Type | Assay |
| --- | --- | --- |
| 0-95 | Massive Basalt | 0 |
| 95-130 | Conglomerate | 0 |
| 130-145 | Amygdaloidal Basalt | 0 |
| 145-220 | Massive Basalt | 0 |
| 220-250 | Amygdaloidal Basalt | 0 |
| 250-360 | Massive Basalt | 0 |
| 360-365 | Conglomerate | 0.8% |
| 365-370 | Conglomerate | 0.8% |
| 370-375 | Conglomerate | 0.6% |
| 375-380 | Conglomerate | 0.5% |
| 380-385 | Conglomerate | 0.4% |
| 385-390 | Conglomerate | 0.6% |
| 390-395 | Conglomerate | 0.7% |
| 395-400 | Conglomerate | 0.7% |
| 400-420 | Amygdaloidal Basalt | 0 |
| 420-555 | Massive Basalt | 0 |
| 555-560 | Amygdaloidal Basalt | 0 |
| 560-605 | Massive Basalt | 0 |
| Bottom of hole |  |  |

DRILLHOLE #7,  67◦ W, 600E

| Depth | Rock Type | Assay |
| --- | --- | --- |
| 0-75 | Massive Basalt | 0 |
| 75-110 | Conglomerate | 0 |
| 110-125 | Amygdaloidal Basalt | 0 |
| 125-210 | Massive Basalt | 0 |
| 210-220 | Amygdaloidal Basalt | 0 |
| 220-320 | Massive Basalt | 0 |
| 320-340 | Conglomerate | 0 |
| 340-350 | Amygdaloidal Basalt | 0 |
| 350-405 | Massive Basalt | 0 |
| 405-420 | Amygdaloidal Basalt | 0 |
| 420-500 | Massive Basalt | 0 |
| 500-505 | Conglomerate | 0.8% |
| 390-395 | Conglomerate | 0.8% |
| 395-400 | Conglomerate | 0.5% |
| 400-420 | Conglomerate | 0.3% |
| 420-555 | Conglomerate | 0.6% |
| 555-560 | Conglomerate | 0.7% |
| 560-605 | Amygdaloidal Basalt | 0 |

DRILLHOLE #6,  60◦ W, 600E

| Depth | Rock Type | Assay |
| --- | --- | --- |
| 0-70 | Massive Basalt | 0 |
| 70-105 | Conglomerate | 0 |
| 105-125 | Amygdaloidal Basalt | 0 |
| 125-205 | Massive Basalt | 0 |
| 205-215 | Amygdaloidal Basalt | 0 |
| 215-305 | Massive Basalt | 0 |
| 305-340 | Conglomerate | 0 |
| 340-355 | Amygdaloidal Basalt | 0 |
| 355-405 | Massive Basalt | 0 |
| 405-425 | Amygdaloidal Basalt | 0 |
| 425-500 | Massive Basalt | 0 |
| 500-505 | Conglomerate | 4.0% |
| 505-510 | Conglomerate | 2.7% |
| 510-515 | Conglomerate | 0.9% |
| 515-520 | Conglomerate | 1.9% |
| 520-525 | Conglomerate | 3.2% |
| 525-540 | Amygdaloidal Basalt | 0 |
| 540-550 | Massive Basalt | 0 |
| Bottom of hole |  |  |

DRILLHOLE #8 Vertical, 600E

| Depth | Rock Type | Assay |
| --- | --- | --- |
| 0-100 | Massive Basalt | 0 |
| 100-150 | Conglomerate | 0 |
| 150-170 | Amygdaloidal Basalt | 0 |
| 170-285 | Massive Basalt | 0 |
| 285-300 | Amygdaloidal Basalt | 0 |
| 300-340 | Massive Basalt | 0 |
| Bottom of hole |  |  |

PLATE #2 Drill Core Data

DRILLHOLE #1, Vertical, 200W

| Depth | Rock Type | Assay |
| --- | --- | --- |
| 0-15 | Conglomerate | 0 |
| 15-40 | Amygdaloidal Basalt | 0 |
| 40-145 | Massive Basalt | 0 |
| 145-165 | Conglomerate | 0 |
| 165-175 | Amygdaloidal Basalt | 0 |
| 175-210 | Massive Basalt | 0 |
| 210-235 | Amygdaloidal Basalt | 0 |
| 235-310 | Massive Basalt | 0 |
| 310-330 | Amygdaloidal Basalt | 0 |
| 330-350 | Massive Basalt | 0 |
| Bottom of hole |  |  |

DRILLHOLE #3,  Vertical W, 120E

| Depth | Rock Type | Assay |
| --- | --- | --- |
| 0-45 | Massive Basalt | 0 |
| 45-70 | Amygdaloidal Basalt | 0 |
| 70-200 | Massive Basalt | 0 |
| 200-235 | Conglomerate | 0 |
| 235-270 | Amygdaloidal Basalt | 0 |
| 270-385 | Massive Basalt | 0 |
| 385-395 | Amygdaloidal Basalt | 1.8% |
| 395-425 | Massive Basalt | 0 |
| Bottom of hole |  |  |

DRILLHOLE #2,  62◦ W, 120E

| Depth | Rock Type | Assay |
| --- | --- | --- |
| 0-35 | Massive Basalt | 0 |
| 35-55 | Amygdaloidal Basalt | 0 |
| 55-145 | Massive Basalt | 0 |
| 145-175 | Conglomerate | 0 |
| 175-200 | Amygdaloidal Basalt | 0 |
| 200-265 | Massive Basalt | 0 |
| 265-275 | Conglomerate | 0 |
| 275-285 | Amygdaloidal Basalt | 0 |
| 285-315 | Massive Basalt | 0 |
| 315-330 | Amygdaloidal Basalt | 0 |
| 330-350 | Massive Basalt | 0 |
| Bottom |  |  |

DRILLHOLE #4 45◦, 600E

| Depth | Rock Type | Assay |
| --- | --- | --- |
| 0-70 | Massive Basalt | 0 |
| 70-105 | Conglomerate | 0 |
| 105-125 | Amygdaloidal Basalt | 0 |
| 125-200 | Massive Basalt | 0 |
| 200-225 | Amygdaloidal Basalt | 0 |
| 225-295 | Massive Basalt | 0 |
| 295-315 | Conglomerate | 0 |
| 315-325 | Amygdaloidal Basalt | 0 |
| 325-375 | Massive Basalt | 0 |
| 375-395 | Amygdaloidal Basalt | 0 |
| 395-470 | Massive Basalt | 0 |
| 470-480 | Conglomerate | 0.5% |
| 480-490 | Conglomerate | 0.4% |
| 490-495 | Conglomerate | 0.6% |
| 495-520 | Amygdaloidal Basalt | 0 |
| 520-600 | Massive Basalt | 0 |
| 600-605 | Amygdaloidal Basalt | 2% |
| 605-640 | Massive Basalt | 0 |
| 640-655 | Amygdaloidal Basalt | 0 |
| 655-680 | Massive Basalt | 0 |

DRILLHOLE #5, 57◦ W,  600E

| Depth | Rock Type | Assay |
| --- | --- | --- |
| 0-70 | Massive Basalt | 0 |
| 70-105 | Conglomerate | 0 |
| 105-125 | Amygdaloidal Basalt | 0 |
| 125-200 | Massive Basalt | 0 |
| 200-230 | Amygdaloidal Basalt | 0 |
| 230-305 | Massive Basalt | 0 |
| 305-325 | Conglomerate | 0 |
| 325-340 | Amygdaloidal Basalt | 0 |
| 340-390 | Massive Basalt | 0 |
| 390-410 | Amygdaloidal Basalt | 0 |
| 410-485 | Conglomerate | 3.2% |
| 485-490 | Conglomerate | 2.6% |
| 490-495 | Conglomerate | 1.7% |
| 495-500 | Conglomerate | 0.7% |
| 500-505 | Conglomerate | 2.0 |
| 505-510 | Conglomerate | 2.6% |
| 510-515 | Conglomerate | 2.8 |
| 515-540 | Amygdaloidal Basalt | 0 |
| 540-625 | Massive Basalt | 0 |
| 625-630 | Amygdaloidal Basalt | 0 |
| 630-665 | Massive Basalt | 0 |
| 665-680 | Amygdaloidal Basalt | 0 |

DRILLHOLE #7,  75◦ W, 600E

| Depth | Rock Type | Assay |
| --- | --- | --- |
| 0-75 | Massive Basalt | 0 |
| 75-110 | Conglomerate | 0 |
| 110-125 | Amygdaloidal Basalt | 0 |
| 125-210 | Massive Basalt | 0 |
| 210-220 | Amygdaloidal Basalt | 0 |
| 220-320 | Massive Basalt | 0 |
| 320-340 | Conglomerate | 0 |
| 340-350 | Amygdaloidal Basalt | 0 |
| 350-405 | Massive Basalt | 0 |
| 405-420 | Amygdaloidal Basalt | 0 |
| 420-500 | Massive Basalt | 0 |
| 500-505 | Conglomerate | 0.8% |
| 390-395 | Conglomerate | 0.8% |
| 395-400 | Conglomerate | 0.5% |
| 400-420 | Conglomerate | 0.3% |
| 420-555 | Conglomerate | 0.6% |
| 555-560 | Conglomerate | 0.7% |
| 560-605 | Amygdaloidal Basalt | 0 |

DRILLHOLE #6,  65◦ W, 600E

| Depth | Rock Type | Assay |
| --- | --- | --- |
| 0-70 | Massive Basalt | 0 |
| 70-105 | Conglomerate | 0 |
| 105-125 | Amygdaloidal Basalt | 0 |
| 125-205 | Massive Basalt | 0 |
| 205-215 | Amygdaloidal Basalt | 0 |
| 215-305 | Massive Basalt | 0 |
| 305-340 | Conglomerate | 0 |
| 340-355 | Amygdaloidal Basalt | 0 |
| 355-405 | Massive Basalt | 0 |
| 405-425 | Amygdaloidal Basalt | 0 |
| 425-500 | Massive Basalt | 0 |
| 500-505 | Conglomerate | 4.0% |
| 505-510 | Conglomerate | 2.7% |
| 510-515 | Conglomerate | 0.9% |
| 515-520 | Conglomerate | 1.9% |
| 520-525 | Conglomerate | 3.2% |
| 525-540 | Amygdaloidal Basalt | 0 |
| 540-550 | Massive Basalt | 0 |
| Bottom of hole |  |  |

DRILLHOLE #8 Vertical, 600E

| Depth | Rock Type | Assay |
| --- | --- | --- |
| 0-100 | Massive Basalt | 0 |
| 100-150 | Conglomerate | 0 |
| 150-165 | Amygdaloidal Basalt | 0 |
| 165-285 | Massive Basalt | 0 |
| 285-315 | Amygdaloidal Basalt | 0 |
| 315-340 | Massive Basalt | 0 |
| Bottom of hole |  |  |